SIMULATION 6

Simulation-Based Investigation of Microwave Semiconductor Devices and Frequency-Conversion Characteristics of a Microwave Mixer

# 1. Aim

To simulate frequency conversion in a microwave mixer, identify the intermediate-frequency (IF) and sum-frequency components, and compare simulated frequencies with theoretical values.

# 2. Objectives

1. To develop simulation models representing the operating characteristics of selected microwave semiconductor devices.

To analyze the electrical characteristics and frequency-dependent behavior of Gunn, Schottky or PIN diode-based microwave structures.

To simulate frequency conversion in a microwave mixer and identify the generated frequency components.

To evaluate conversion characteristics and identify suitable operating conditions for microwave applications.

# 3. Input Parameters

| S. No. | Input parameter | Symbol / setting | Value to enter |
| --- | --- | --- | --- |
| 1 | RF frequency | fRF | 10 GHz |
| 2 | LO frequency | fLO | 9.5 GHz |
| 3 | RF input power | PRF | −10 dBm |
| 4 | LO power | PLO | +7 dBm |
| 5 | IF load | R | 50 Ω |
| 6 | Sampling frequency | Fs | 100 GHz |
| 7 | Number of samples | N | 4000 |

# 4. Code

clc;

clear;

close all;

% ECA13 - Microwave Engineering

% Microwave Mixer Frequency Conversion Simulation

%% Input Parameters

fRF = 10e9;             % RF frequency = 10 GHz

fLO = 9.5e9;            % LO frequency = 9.5 GHz

PRF = -10;              % RF power in dBm

PLO = 7;                % LO power in dBm

R = 50;                 % IF load in ohms

%% Theoretical Frequency Calculation

fIF = abs(fRF - fLO);

fSUM = fRF + fLO;

%% Convert Input Power to Voltage

PRF_W = 10^((PRF-30)/10);

PLO_W = 10^((PLO-30)/10);

VRF = sqrt(2*PRF_W*R);

VLO = sqrt(2*PLO_W*R);

%% Generate RF and LO Signals

Fs = 100e9;             % Sampling frequency

N = 4000;

t = (0:N-1)/Fs;

RF = VRF*cos(2*pi*fRF*t);

LO = VLO*cos(2*pi*fLO*t);

%% Nonlinear Mixer Model

% Product model produces difference and sum frequencies

Vout = RF .* LO;

%% Calculate Output Spectrum using FFT

Y = fft(Vout);

A = abs(Y/N);

A = A(1:N/2+1);

A(2:end-1) = 2*A(2:end-1);

f = (0:N/2)*Fs/N;

% Normalize spectrum

AdB = 20*log10(A/max(A) + eps);

%% Identify Simulated Frequency Components

idxIF = find(f >= 0.3e9 & f <= 0.7e9);

idxSUM = find(f >= 19.3e9 & f <= 19.7e9);

[~,i1] = max(A(idxIF));

[~,i2] = max(A(idxSUM));

p1 = idxIF(i1);

p2 = idxSUM(i2);

fIFsim = f(p1);

fSUMsim = f(p2);

%% Percentage Frequency Deviation

devIF = abs(fIFsim-fIF)/fIF*100;

devSUM = abs(fSUMsim-fSUM)/fSUM*100;

%% Display Results

fprintf('\nMICROWAVE MIXER FREQUENCY CONVERSION\n');

fprintf('RF Frequency              = %.2f GHz\n',fRF/1e9);

fprintf('LO Frequency              = %.2f GHz\n',fLO/1e9);

fprintf('RF Power                  = %.2f dBm\n',PRF);

fprintf('LO Power                  = %.2f dBm\n',PLO);

fprintf('IF Load                   = %.0f ohms\n',R);

fprintf('\nTHEORETICAL FREQUENCIES\n');

fprintf('IF Frequency              = %.3f GHz\n',fIF/1e9);

fprintf('Sum Frequency             = %.3f GHz\n',fSUM/1e9);

fprintf('\nSIMULATED FREQUENCIES\n');

fprintf('Simulated IF              = %.3f GHz\n',fIFsim/1e9);

fprintf('Simulated Sum             = %.3f GHz\n',fSUMsim/1e9);

fprintf('\nFREQUENCY DEVIATION\n');

fprintf('IF Deviation              = %.3f %%\n',devIF);

fprintf('Sum Frequency Deviation   = %.3f %%\n',devSUM);

%% Plot All Results in One Figure

figure('Color','w');

plot(f/1e9,AdB,'b','LineWidth',1.5);

hold on;

% Theoretical output frequencies

xline(fIF/1e9,'r--','Theoretical IF');

xline(fSUM/1e9,'r--','Theoretical Sum');

% RF and LO input frequency references

xline(fLO/1e9,'k:','LO Input');

xline(fRF/1e9,'m:','RF Input');

% Mark simulated output peaks

plot(fIFsim/1e9,AdB(p1),'ro','MarkerSize',8,...

'MarkerFaceColor','r');

plot(fSUMsim/1e9,AdB(p2),'ro','MarkerSize',8,...

'MarkerFaceColor','r');

xlabel('Frequency (GHz)');

ylabel('Normalized Magnitude (dB)');

title('Microwave Mixer Frequency Conversion Spectrum');

info = sprintf(['RF = %.1f GHz, LO = %.1f GHz\n' ...

'RF Power = %.0f dBm, LO Power = +%.0f dBm\n' ...

'Theoretical IF = %.2f GHz, Simulated IF = %.2f GHz\n' ...

'Theoretical Sum = %.2f GHz, Simulated Sum = %.2f GHz\n' ...

'IF Deviation = %.2f%%, Sum Deviation = %.2f%%'], ...

fRF/1e9,fLO/1e9,PRF,PLO,...

fIF/1e9,fIFsim/1e9,fSUM/1e9,fSUMsim/1e9,...

devIF,devSUM);

text(0.98,0.97,info,'Units','normalized',...

'VerticalAlignment','top','HorizontalAlignment','right',...

'BackgroundColor','white','EdgeColor','black',...

'FontSize',9);

grid on;

xlim([0 25]);

ylim([-80 5]);

hold off;

# 5. Calculated Results (From MATLAB Code)

| Output parameter | Theoretical value | Simulated value | % deviation |
| --- | --- | --- | --- |
| Difference / IF frequency | 0.500 GHz | 0.500 GHz | 0.000% |
| Sum frequency | 19.500 GHz | 19.500 GHz | 0.000% |
| RF input frequency | 10.000 GHz | 10.000 GHz (input reference) | — |
| LO input frequency | 9.500 GHz | 9.500 GHz (input reference) | — |

# 6. Output Parameters

| Output parameter | Expected / theoretical value | Value recorded from supplied code |
| --- | --- | --- |
| IF frequency | ¦fRF − fLO¦ = 0.5 GHz | 0.500 GHz |
| Sum frequency | fRF + fLO = 19.5 GHz | 19.500 GHz |
| Dominant conversion products | IF and sum components | Both peaks identified in spectrum |
| Normalized spectral peak level | Peak normalized to 0 dB | Approximately 0 dB at each identified peak |
| IF output power | Requires calibrated mixer model or measurement | Not calculated by supplied script |
| Conversion loss | CL = PRF − PIF | Not calculated by supplied script |