SIMULATION 6

Simulation-Based Investigation of Microwave Semiconductor Devices and Frequency-Conversion Characteristics of a Microwave Mixer

# 1. Aim

To analyze microwave mixer conversion performance using the lab-manual RF/LO settings, estimate the IF output power from the assumed 8 dB conversion-loss model, and compare the result with the expected passive-mixer range of 6–10 dB.

# 2. Objectives

1. To develop simulation models representing the operating characteristics of selected microwave semiconductor devices.

To analyze the electrical characteristics and frequency-dependent behavior of Gunn, Schottky or PIN diode-based microwave structures.

To simulate frequency conversion in a microwave mixer and identify the generated frequency components.

To evaluate conversion characteristics and identify suitable operating conditions for microwave applications.

# 3. Input Parameters

| S. No. | Input parameter | Symbol / setting | Value (PDF / code) |
| --- | --- | --- | --- |
| 1 | RF frequency | fRF | 10 GHz |
| 2 | LO frequency | fLO | 9.5 GHz |
| 3 | RF input power | PRF | −10 dBm |
| 4 | LO power | PLO | +7 dBm |
| 5 | IF load | R (ZIF in manual) | 50 Ω |
| 6 | Assumed nominal conversion loss | CL | 8 dB (illustrative model) |
| 7 | Sampling frequency | Fs | 5 GHz (code setting) |
| 8 | Number of samples | N | 5000 (code setting) |

# 4. Code

clc;

clear;

close all;

% ECA13 - Microwave Engineering

% Microwave Mixer Conversion Loss Analysis

%% Input Parameters

fRF = 10e9;             % RF frequency (Hz)

fLO = 9.5e9;            % LO frequency (Hz)

PRF = -10;              % RF power (dBm)

PLO = 7;                % LO power (dBm)

R = 50;                 % IF load (ohms)

%% Theoretical IF Frequency

fIF = abs(fRF-fLO);

%% Behavioral Mixer Model

% Assumed nominal conversion loss for illustration

CL = 8;                 % Conversion loss (dB)

PIF = PRF-CL;           % IF output power (dBm)

% Power conversion: dBm to watts

PIF_W = 10^((PIF-30)/10);

% IF sinusoidal signal across 50-ohm load

Vpeak = sqrt(2*PIF_W*R);

%% Generate IF Signal and Spectrum

Fs = 5e9;

N = 5000;

t = (0:N-1)/Fs;

vIF = Vpeak*cos(2*pi*fIF*t);

Y = fft(vIF);

A = abs(Y/N);

A = A(1:N/2+1);

A(2:end-1) = 2*A(2:end-1);

f = (0:N/2)*Fs/N;

% Convert spectral voltage amplitude to power in dBm

Pspectrum = (A/sqrt(2)).^2/R;

PdBm = 10*log10(Pspectrum/1e-3 + eps);

% Locate simulated IF peak

[~,idx] = max(A);

fIFsim = f(idx);

% Calculate conversion loss from simulated IF power

PIFsim = 10*log10(((A(idx)/sqrt(2))^2/R)/1e-3);

CLsim = PRF-PIFsim;

% Frequency deviation

devIF = abs(fIFsim-fIF)/fIF*100;

%% Conversion Performance versus LO Power

LOsweep = 0:1:13;

% Illustrative assumed loss curve, not measured device data

CLsweep = interp1([0 3 7 10 13], ...

[12 10 8 7.5 7.5], LOsweep);

%% Conversion Performance versus RF Power

RFsweep = -20:2:0;

PIFsweep = RFsweep-CL;

%% Display Results

fprintf('\nMICROWAVE MIXER CONVERSION LOSS\n');

fprintf('RF Frequency          = %.2f GHz\n',fRF/1e9);

fprintf('LO Frequency          = %.2f GHz\n',fLO/1e9);

fprintf('IF Frequency          = %.2f GHz\n',fIF/1e9);

fprintf('RF Input Power        = %.2f dBm\n',PRF);

fprintf('LO Input Power        = +%.2f dBm\n',PLO);

fprintf('IF Load               = %.0f ohms\n',R);

fprintf('Simulated IF Frequency= %.3f GHz\n',fIFsim/1e9);

fprintf('IF Output Power       = %.2f dBm\n',PIFsim);

fprintf('Conversion Loss       = %.2f dB\n',CLsim);

fprintf('Frequency Deviation   = %.3f %%\n',devIF);

if CLsim >= 6 && CLsim <= 10

fprintf('Result: Within expected 6-10 dB range.\n');

else

fprintf('Result: Outside expected 6-10 dB range.\n');

end

%% Plot All Results in One Figure

figure('Color','w');

tiledlayout(2,2,'TileSpacing','compact','Padding','compact');

% 1. IF Output Spectrum

nexttile;

plot(f/1e9,PdBm,'b','LineWidth',1.4);

hold on;

plot(fIFsim/1e9,PIFsim,'ro','MarkerFaceColor','r');

xline(fIF/1e9,'k--','Theoretical IF');

xlabel('Frequency (GHz)');

ylabel('Power (dBm)');

title('IF Output Spectrum');

xlim([0 1.5]);

grid on;

% 2. Conversion Loss Result

nexttile;

bar(1,CLsim);

hold on;

yline(6,'g--','6 dB');

yline(10,'r--','10 dB');

xlim([0 2]);

xticks(1);

xticklabels({'Simulated'});

ylabel('Conversion Loss (dB)');

title(sprintf('CL = %.2f dB',CLsim));

grid on;

% 3. Effect of LO Power

nexttile;

plot(LOsweep,CLsweep,'m-o','LineWidth',1.5);

hold on;

plot(PLO,CL,'ks','MarkerFaceColor','y','MarkerSize',8);

xlabel('LO Power (dBm)');

ylabel('Conversion Loss (dB)');

title('Effect of LO Power');

legend('Illustrative model','Operating point','Location','best');

grid on;

% 4. Effect of RF Input Power

nexttile;

plot(RFsweep,PIFsweep,'b-o','LineWidth',1.5);

hold on;

plot(PRF,PIF,'rs','MarkerFaceColor','r','MarkerSize',8);

xlabel('RF Input Power (dBm)');

ylabel('IF Output Power (dBm)');

title('Effect of RF Power');

legend('Illustrative model','Operating point','Location','best');

grid on;

sgtitle('Microwave Mixer Conversion Loss Analysis');

# 5. Calculated Results

| Output parameter | Theoretical / expected value (manual) | Value from supplied code | % deviation / status |
| --- | --- | --- | --- |
| Difference frequency (IF) | ¦10 − 9.5¦ = 0.500 GHz | 0.500 GHz | 0.000% (≤ 2% limit) |
| Sum frequency | 10 + 9.5 = 19.500 GHz | Not simulated by this code | Cannot calculate |
| IF output power | Record IF power at 0.5 GHz | −18.00 dBm (model estimate) | — |
| Conversion loss | 6–10 dB expected for passive mixer | 8.00 dB (assumed-model result) | Within expected range |
| Dominant conversion product | 0.5 GHz IF; other spurs ≤ −20 dBc | 0.5 GHz IF signal generated; spurs not modeled | Partial check only |
| LO-power sweep | Interpret effect of LO power | Illustrative curve, 0–13 dBm | Not measured device data |
| RF-power sweep | Interpret effect of RF input power | Illustrative curve, −20 to 0 dBm | Not measured device data |

# 6. Output Parameters

| Output parameter | Expected / theoretical value | Value recorded from supplied code |
| --- | --- | --- |
| IF frequency | ¦fRF − fLO¦ = 0.5 GHz | 0.500 GHz |
| Sum frequency | fSUM = fRF + fLO = 19.5 GHz | Not generated by this code |
| IF output power | PIF = PRF − CL = −18 dBm | −18.00 dBm (behavioral model) |
| Conversion loss | CL = PRF − PIF; expected 6–10 dB | 8.00 dB; within expected range |
| IF frequency deviation | ¦fIFsim − fIF¦ / fIF × 100% | 0.000% |
| LO-power sweep | Illustrative conversion-loss curve | 0–13 dBm; illustrative only |
| RF-power sweep | Illustrative IF output-power curve | −20 to 0 dBm; illustrative only |
| Spurious components | Other spurs should be ≤ −20 dBc | Not modeled by supplied code |