EXPERIMENT 2

Numerical Simulation and Performance Analysis of Microwave Transmission Lines and Two-Port Networks Using Scattering Parameters

# 1. Aim

To model a 50 Ω microwave two-port transmission line over an 8–12 GHz frequency range, determine its S-parameters, and evaluate return loss, insertion loss and transmission efficiency at the design frequency of 10 GHz.

# 2. Objectives

1. To model coaxial, waveguide or microstrip transmission structures for microwave-frequency operation.

To determine characteristic impedance, propagation constant and transmission characteristics of the selected structure.

To obtain and analyze S11 , S21 , S12 and S22 over a specified frequency range.

To evaluate return loss, insertion loss and transmission efficiency from the simulated S-parameter response.

# 3. Parameters Used

| Parameter | Symbol / Setting | Value |
| --- | --- | --- |
| Design / centre frequency | f0 | 10 GHz |
| Reference impedance | Z0 | 50 Ω |
| Frequency sweep | fstart–fstop | 8–12 GHz |
| Frequency step | Δf | 0.01 GHz (10 MHz) |
| Number of frequency points | N | 401 |
| Transmission-line length | L | 50 mm |
| Speed of light | c | 3 × 10⁸ m/s |
| Characteristic impedance model | Zc | 50 + 5((f − f0)/(2 GHz))² Ω |
| Attenuation model | αdB/m | 0.5 + 0.5((f − f0)/(2 GHz))² dB/m |
| Port reference impedance | Z0 | 50 Ω at both ports |

# 4. Code

clc;

clear;

close all;

%% EXPERIMENT 2: MICROWAVE TWO-PORT NETWORK

% 50 Ohm transmission line, 8-12 GHz frequency sweep

%% 1. INPUT PARAMETERS

c = 3e8;          % Speed of light (m/s)

Z0 = 50;          % Reference impedance (Ohms)

L = 50e-3;        % Transmission line length (m)

f0 = 10e9;        % Center frequency (Hz)

f = 8e9:0.01e9:12e9;

fGHz = f / 1e9;

%% 2. TRANSMISSION LINE MODEL

% Illustrative model: physical line geometry and material

% properties are not specified in the experiment sheet.

Zc = 50 + 5 * ((f - f0) / (2e9)).^2;

alpha_dB_per_m = 0.5 + ...

0.5 * ((f - f0) / (2e9)).^2;

alpha = alpha_dB_per_m * log(10) / 20;

beta = 2 * pi * f / c;

gamma = alpha + 1j * beta;

%% 3. ABCD PARAMETERS

A = cosh(gamma * L);

B = Zc .* sinh(gamma * L);

C = sinh(gamma * L) ./ Zc;

D = A;

den = A + B / Z0 + C * Z0 + D;

%% 4. S-PARAMETERS

S11 = (A + B / Z0 - C * Z0 - D) ./ den;

S21 = 2 ./ den;

S12 = 2 * (A .* D - B .* C) ./ den;

S22 = (-A + B / Z0 - C * Z0 + D) ./ den;

% Convert S-parameters to dB

S11_dB = 20 * log10(max(abs(S11), 1e-12));

S21_dB = 20 * log10(max(abs(S21), 1e-12));

S12_dB = 20 * log10(max(abs(S12), 1e-12));

S22_dB = 20 * log10(max(abs(S22), 1e-12));

%% 5. PERFORMANCE PARAMETERS

RL = -20 * log10(max(abs(S11), 1e-12));

IL = -20 * log10(max(abs(S21), 1e-12));

eta = abs(S21).^2 * 100;

%% 6. DISPLAY RESULTS AT 10 GHz

[~, idx10] = min(abs(f - f0));

fprintf('\\nMICROWAVE ENGINEERING - EXPERIMENT 2\\n');

fprintf('-----------------------------------\\n');

fprintf('Operating frequency = %.2f GHz\\n', fGHz(idx10));

fprintf('Reference impedance = %.2f Ohms\\n', Z0);

fprintf('Line length = %.2f mm\\n', L * 1000);

fprintf('Characteristic impedance = %.3f Ohms\\n', Zc(idx10));

fprintf('Propagation constant beta = %.3f rad/m\\n', beta(idx10));

fprintf('S11 = %.4f dB\\n', S11_dB(idx10));

fprintf('S21 = %.4f dB\\n', S21_dB(idx10));

fprintf('S12 = %.4f dB\\n', S12_dB(idx10));

fprintf('S22 = %.4f dB\\n', S22_dB(idx10));

fprintf('Return Loss = %.4f dB\\n', RL(idx10));

fprintf('Insertion Loss = %.4f dB\\n', IL(idx10));

fprintf('Transmission Efficiency = %.4f %%\\n', eta(idx10));

%% 7. OPTIMUM FREQUENCY

[~, idxMinS11] = min(abs(S11));

[~, idxMaxS21] = max(abs(S21));

fprintf('\\nOPTIMUM FREQUENCY RESULTS\\n');

fprintf('Minimum |S11| frequency = %.2f GHz\\n', fGHz(idxMinS11));

fprintf('S11 at minimum = %.4f dB\\n', S11_dB(idxMinS11));

fprintf('S21 at minimum = %.4f dB\\n', S21_dB(idxMinS11));

fprintf('Return Loss = %.4f dB\\n', RL(idxMinS11));

fprintf('Insertion Loss = %.4f dB\\n', IL(idxMinS11));

fprintf('Maximum |S21| frequency = %.2f GHz\\n', fGHz(idxMaxS21));

fprintf('S11 at maximum transmission = %.4f dB\\n', S11_dB(idxMaxS21));

fprintf('S21 at maximum transmission = %.4f dB\\n', S21_dB(idxMaxS21));

fprintf('Return Loss = %.4f dB\\n', RL(idxMaxS21));

fprintf('Insertion Loss = %.4f dB\\n', IL(idxMaxS21));

fprintf('Efficiency = %.4f %%\\n', eta(idxMaxS21));

%% 8. GRAPH 1 TO GRAPH 9 USING SUBPLOT

figure('Color', 'w', 'Position', [50 50 1400 850]);

subplot(3, 3, 1);

plot(fGHz, S11_dB, 'b', 'LineWidth', 1.5);

xlabel('Frequency (GHz)'); ylabel('|S11| (dB)');

title('Input Reflection S11'); grid on; xlim([8 12]);

subplot(3, 3, 2);

plot(fGHz, S21_dB, 'r', 'LineWidth', 1.5);

xlabel('Frequency (GHz)'); ylabel('|S21| (dB)');

title('Forward Transmission S21'); grid on; xlim([8 12]);

subplot(3, 3, 3);

plot(fGHz, S12_dB, 'm', 'LineWidth', 1.5);

xlabel('Frequency (GHz)'); ylabel('|S12| (dB)');

title('Reverse Transmission S12'); grid on; xlim([8 12]);

subplot(3, 3, 4);

plot(fGHz, S22_dB, 'k', 'LineWidth', 1.5);

xlabel('Frequency (GHz)'); ylabel('|S22| (dB)');

title('Output Reflection S22'); grid on; xlim([8 12]);

subplot(3, 3, 5);

plot(fGHz, RL, 'b', 'LineWidth', 1.5);

xlabel('Frequency (GHz)'); ylabel('Return Loss (dB)');

title('Return Loss'); grid on; xlim([8 12]);

subplot(3, 3, 6);

plot(fGHz, IL, 'r', 'LineWidth', 1.5);

xlabel('Frequency (GHz)'); ylabel('Insertion Loss (dB)');

title('Insertion Loss'); grid on; xlim([8 12]);

subplot(3, 3, 7);

plot(fGHz, eta, 'g', 'LineWidth', 1.5);

xlabel('Frequency (GHz)'); ylabel('Efficiency (%)');

title('Transmission Efficiency'); grid on; xlim([8 12]);

subplot(3, 3, 8);

plot(fGHz, beta, 'Color', [0.5 0.2 0.7], 'LineWidth', 1.5);

xlabel('Frequency (GHz)'); ylabel('\\beta (rad/m)');

title('Propagation Constant'); grid on; xlim([8 12]);

subplot(3, 3, 9);

plot(fGHz, Zc, 'Color', [0.85 0.4 0.1], 'LineWidth', 1.5);

hold on; yline(Z0, '--k', '50 Ohm Reference');

xlabel('Frequency (GHz)'); ylabel('Zc (Ohms)');

title('Characteristic Impedance'); grid on; xlim([8 12]);

sgtitle('Microwave Two-Port Network: 8-12 GHz');

%% 9. IDEAL MATCHED REFERENCE

fprintf('\\nIDEAL MATCHED REFERENCE\\n');

fprintf('S11 = 0 reflection (ideally -Inf dB)\\n');

fprintf('S22 = 0 reflection (ideally -Inf dB)\\n');

fprintf('S21 = 0 dB; S12 = 0 dB\\n');

fprintf('Return Loss = Infinite\\n');

fprintf('Insertion Loss = 0 dB\\n');

fprintf('Efficiency = 100 %%\\n');

%% 10. CONCLUSION

fprintf('\\nCONCLUSION\\n');

fprintf('S-parameters were evaluated over 8-12 GHz.\\n');

fprintf('Return loss, insertion loss and efficiency were calculated.\\n');

fprintf('The optimum frequencies were identified from the sweep.\\n');

fprintf('This is an illustrative model, not validated EM-solver data.\\n');

%% 11. DISPLAY SELECTED SWEEP RESULTS

Results = table(fGHz(:), S11_dB(:), S21_dB(:), S12_dB(:), ...

S22_dB(:), RL(:), IL(:), eta(:), ...

'VariableNames', {'Frequency_GHz','S11_dB','S21_dB', ...

'S12_dB','S22_dB','ReturnLoss_dB','InsertionLoss_dB', ...

'Efficiency_percent'});

disp(Results(1:20:end, :));

# 5. Calculated Results

| Parameter | Value at 10 GHz | Interpretation |
| --- | --- | --- |
| Characteristic impedance Zc | 50.000 Ω | Equal to the 50 Ω reference impedance at centre frequency. |
| Propagation constant β | 209.440 rad/m | Phase constant calculated from β = 2πf/c. |
| S11 | −240.000 dB (numerical floor) | Reflection is effectively zero at 10 GHz in this model. |
| S21 | −0.025 dB | Forward transmission is close to 0 dB. |
| S12 | −0.025 dB | Reverse transmission equals forward transmission for this reciprocal model. |
| S22 | −240.000 dB (numerical floor) | Output reflection is effectively zero at 10 GHz in this model. |
| Return loss | 240.000 dB (numerical floor) | Very good matching; value is limited by the 10⁻¹² magnitude floor in the code. |
| Insertion loss | 0.025 dB | Small forward transmission loss. |
| Transmission efficiency | 99.426% | Calculated using η = ¦S21¦² × 100%. |

# 6. Output Parameters

| Output parameter | Value at 10 GHz | Interpretation |
| --- | --- | --- |
| Input reflection S11 | −240.000 dB (numerical floor) | Matched input in the illustrative model. |
| Forward transmission S21 | −0.025 dB | Nearly all incident power is transmitted, subject to the model loss. |
| Reverse transmission S12 | −0.025 dB | Reciprocal transmission response. |
| Output reflection S22 | −240.000 dB (numerical floor) | Matched output in the illustrative model. |
| Return loss | 240.000 dB (numerical floor) | Higher return loss indicates better matching. |
| Insertion loss | 0.025 dB | Low modeled transmission loss at 10 GHz. |
| Transmission efficiency | 99.426% | Estimated transmitted power percentage from ¦S21¦². |