EXPERIMENT 1

TEM, TE and TM Mode Analysis using MATLAB

# 1. Aim

To visualize illustrative TEM, TE and TM electromagnetic field patterns, calculate cutoff frequencies and propagation constants, and compare the propagation status of selected modes at an operating frequency of 10 GHz.

# 2. Objectives

Compare illustrative TEM, TE10, TE20 and TM11 field patterns.

Visualize electric- and magnetic-field distributions.

Calculate cutoff frequencies and plot propagation constant versus frequency.

Identify modes that are propagating or below cutoff at 10 GHz.

# 3. Parameters Used

| Parameter | Value |
| --- | --- |
| Speed of light, c | 3 × 10^8 m/s |
| Waveguide width, a | 0.02 m |
| Waveguide height, b | 0.01 m |
| Operating frequency | 10 GHz |
| Frequency sweep | 1–30 GHz |

# 4. Software Required

MATLAB. No additional toolbox is required for the basic plots.

# 5. MATLAB Code

Save the following complete script as Experiment1.m and run it in MATLAB. The program creates one figure containing nine graphs and prints numerical results in the Command Window.

clc;

clear;

close all;

%% MICROWAVE ENGINEERING - EXPERIMENT 1

% TEM, TE and TM Mode Analysis

% All Four Objectives in ONE Figure

%% CONSTANTS AND WAVEGUIDE DIMENSIONS

c = 3e8;

% Rectangular waveguide dimensions

a = 0.02;       % Width in metres

b = 0.01;       % Height in metres

% Operating frequency

f0 = 10e9;      % 10 GHz

k0 = 2*pi*f0/c;

%% SPATIAL GRID

x = linspace(0,a,60);

y = linspace(0,b,60);

[X,Y] = meshgrid(x,y);

%% OBJECTIVE 1: ELECTROMAGNETIC MODE COMPARISON

% Normalized illustrative electric-field patterns

E_TE10 = sin(pi*X/a);

E_TE20 = sin(2*pi*X/a);

E_TM11 = sin(pi*X/a).*sin(pi*Y/b);

% TEM field illustration (two-conductor structure)

r = linspace(1,10,60);

theta = linspace(0,2*pi,100);

[R,THETA] = meshgrid(r,theta);

X_TEM = R.*cos(THETA);

Y_TEM = R.*sin(THETA);

E_TEM = 1./R;

%% OBJECTIVE 2: ELECTRIC AND MAGNETIC FIELD DISTRIBUTIONS

% Illustrative normalized field patterns

E_field = E_TE10;

H_field = abs(cos(pi*X/a));

%% OBJECTIVE 3: CUTOFF FREQUENCIES

fc_TEM = 0;

fc_TE10 = c/(2*a);

fc_TE20 = c/a;

fc_TM11 = (c/2)*sqrt((1/a)^2 + (1/b)^2);

%% PROPAGATION CONSTANTS

beta_TEM = k0;

% TE10 propagation constant

if f0 > fc_TE10

beta_TE10 = k0*sqrt(1-(fc_TE10/f0)^2);

else

beta_TE10 = NaN;

end

% TE20 propagation constant

if f0 > fc_TE20

beta_TE20 = k0*sqrt(1-(fc_TE20/f0)^2);

else

beta_TE20 = NaN;

end

% TM11 propagation constant

if f0 > fc_TM11

beta_TM11 = k0*sqrt(1-(fc_TM11/f0)^2);

else

beta_TM11 = NaN;

end

%% FREQUENCY SWEEP

freq = linspace(1e9,30e9,500);

k = 2*pi*freq/c;

betaTEM = k;

betaTE10 = NaN(size(freq));

idx = freq > fc_TE10;

betaTE10(idx) = sqrt(k(idx).^2-(2*pi*fc_TE10/c)^2);

betaTE20 = NaN(size(freq));

idx = freq > fc_TE20;

betaTE20(idx) = sqrt(k(idx).^2-(2*pi*fc_TE20/c)^2);

betaTM11 = NaN(size(freq));

idx = freq > fc_TM11;

betaTM11(idx) = sqrt(k(idx).^2-(2*pi*fc_TM11/c)^2);

%% OBJECTIVE 4: PROPAGATION STATUS AT OPERATING FREQUENCY

% 1 = propagating, 0 = below cutoff

status = [1, f0 > fc_TE10, f0 > fc_TE20, f0 > fc_TM11];

%% COMBINE ALL GRAPHS IN ONE FIGURE

figure('Name','Microwave Engineering - All Objectives', ...

'Color','w','Position',[50 50 1500 900]);

%% GRAPH 1: TE10 MODE

subplot(3,3,1);

surf(X*1000,Y*1000,E_TE10);

shading interp;

xlabel('X (mm)');

ylabel('Y (mm)');

zlabel('Normalized E-field');

title('Objective 1: TE_{10} Mode');

colorbar;

view(45,30);

grid on;

%% GRAPH 2: TE20 MODE

subplot(3,3,2);

surf(X*1000,Y*1000,E_TE20);

shading interp;

xlabel('X (mm)');

ylabel('Y (mm)');

zlabel('Normalized E-field');

title('Objective 1: TE_{20} Mode');

colorbar;

view(45,30);

grid on;

%% GRAPH 3: TM11 MODE

subplot(3,3,3);

surf(X*1000,Y*1000,E_TM11);

shading interp;

xlabel('X (mm)');

ylabel('Y (mm)');

zlabel('Normalized E-field');

title('Objective 1: TM_{11} Mode');

colorbar;

view(45,30);

grid on;

%% GRAPH 4: TEM MODE

subplot(3,3,4);

surf(X_TEM,Y_TEM,E_TEM);

shading interp;

xlabel('X (arbitrary units)');

ylabel('Y (arbitrary units)');

zlabel('Normalized E-field');

title('Objective 1: TEM Mode');

colorbar;

view(45,30);

grid on;

%% GRAPH 5: ELECTRIC FIELD DISTRIBUTION

subplot(3,3,5);

imagesc(x*1000,y*1000,E_field);

set(gca,'YDir','normal');

xlabel('X (mm)');

ylabel('Y (mm)');

title('Objective 2: Electric Field (TE_{10})');

colorbar;

axis tight;

%% GRAPH 6: MAGNETIC FIELD DISTRIBUTION

subplot(3,3,6);

imagesc(x*1000,y*1000,H_field);

set(gca,'YDir','normal');

xlabel('X (mm)');

ylabel('Y (mm)');

title('Objective 2: Magnetic Field (Illustrative)');

colorbar;

axis tight;

%% GRAPH 7: PROPAGATION CONSTANT VS FREQUENCY

subplot(3,3,7);

plot(freq/1e9,betaTEM,'LineWidth',1.5);

hold on;

plot(freq/1e9,betaTE10,'LineWidth',1.5);

plot(freq/1e9,betaTE20,'LineWidth',1.5);

plot(freq/1e9,betaTM11,'LineWidth',1.5);

xline(fc_TE10/1e9,'--','TE_{10} cutoff');

xline(fc_TE20/1e9,'--','TE_{20} cutoff');

xline(fc_TM11/1e9,'--','TM_{11} cutoff');

xlabel('Frequency (GHz)');

ylabel('\beta (rad/m)');

title('Objective 3: Propagation Constant');

legend('TEM','TE_{10}','TE_{20}','TM_{11}', ...

'Location','northwest');

grid on;

xlim([1 30]);

%% GRAPH 8: CUTOFF FREQUENCY COMPARISON

subplot(3,3,8);

cutoffGHz = [fc_TEM fc_TE10 fc_TE20 fc_TM11]/1e9;

bar(cutoffGHz);

set(gca,'XTick',1:4);

set(gca,'XTickLabel',{'TEM','TE10','TE20','TM11'});

ylabel('Cutoff Frequency (GHz)');

title('Objective 3: Cutoff Frequencies');

grid on;

%% GRAPH 9: MODE PROPAGATION SUITABILITY

subplot(3,3,9);

bar(double(status));

set(gca,'XTick',1:4);

set(gca,'XTickLabel',{'TEM','TE10','TE20','TM11'});

ylim([0 1.2]);

yticks([0 1]);

yticklabels({'Below cutoff','Propagating'});

ylabel('Propagation Status');

title('Objective 4: Mode Suitability at 10 GHz');

grid on;

sgtitle('ECA13: TEM, TE and TM Electromagnetic Propagation Analysis');

%% DISPLAY NUMERICAL RESULTS

fprintf('\n========================================\n');

fprintf(' MICROWAVE ENGINEERING - EXPERIMENT 1\n');

fprintf('========================================\n');

fprintf('Operating Frequency = %.2f GHz\n',f0/1e9);

fprintf('\nCUTOFF FREQUENCIES\n');

fprintf('TEM  = 0 GHz (ideal two-conductor line)\n');

fprintf('TE10 = %.3f GHz\n',fc_TE10/1e9);

fprintf('TE20 = %.3f GHz\n',fc_TE20/1e9);

fprintf('TM11 = %.3f GHz\n',fc_TM11/1e9);

fprintf('\nPROPAGATION CONSTANTS AT %.2f GHz\n',f0/1e9);

fprintf('TEM  beta = %.3f rad/m\n',beta_TEM);

fprintf('TE10 beta = %.3f rad/m\n',beta_TE10);

if isnan(beta_TE20)

fprintf('TE20 is below cutoff at %.2f GHz\n',f0/1e9);

else

fprintf('TE20 beta = %.3f rad/m\n',beta_TE20);

end

if isnan(beta_TM11)

fprintf('TM11 is below cutoff at %.2f GHz\n',f0/1e9);

else

fprintf('TM11 beta = %.3f rad/m\n',beta_TM11);

end

fprintf('\nPROPAGATION STATUS AT %.2f GHz\n',f0/1e9);

if f0 > fc_TE10

fprintf('TE10: Propagating\n');

else

fprintf('TE10: Below cutoff\n');

end

if f0 > fc_TE20

fprintf('TE20: Propagating\n');

else

fprintf('TE20: Below cutoff\n');

end

if f0 > fc_TM11

fprintf('TM11: Propagating\n');

else

fprintf('TM11: Below cutoff\n');

end

fprintf('\nAll four objectives are represented in one figure.\n');

# 6. Calculated Results (From MATLAB Code)

## Table 1: Cutoff Frequencies

| Mode | Calculation used in code | Cutoff frequency |
| --- | --- | --- |
| TEM | Assumed ideal two-conductor line | 0 GHz |
| TE₁₀ | c / (2a) | 7.500 GHz |
| TE₂₀ | c / a | 15.000 GHz |
| TM₁₁ | (c/2) × √[(1/a)² + (1/b)²] | 16.771 GHz |

## Table 2: Propagation Constants at 10 GHz

| Mode | Value from code | Interpretation |
| --- | --- | --- |
| TEM | 209.440 rad/m | Illustrative ideal two-conductor line |
| TE₁₀ | 138.531 rad/m | Above cutoff; propagating |
| TE₂₀ | N/A (NaN in code) | Below cutoff at 10 GHz |
| TM₁₁ | N/A (NaN in code) | Below cutoff at 10 GHz |

## Table 3: Propagation Status at 10 GHz

| Mode | Cutoff frequency | Status at 10 GHz |
| --- | --- | --- |
| TEM | 0 GHz | Propagating (ideal two-conductor assumption) |
| TE₁₀ | 7.500 GHz | Propagating |
| TE₂₀ | 15.000 GHz | Below cutoff |
| TM₁₁ | 16.771 GHz | Below cutoff |